SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO5: 
Design network topologies star, mesh, bus, and hybrid using networking devices, transmission media, and cabling standards

(BL2,BL5)

Assessment Tool Weightage: 40%

QUESTIONS: 3								Total Marks:30

Annexure A

Constraint-Based Problem Solving

Code of Conduct:

I Sri Sayee K(192421062) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

ksrisayee

Annexure A

Constraint-Based Problem Solving

- A  multinational company is opening a new branch office in a remote rural location where high-speed wired internet is unavailable. The office must support cloud-based applications, video conferencing, and secure communication with the headquarters. The company has a limited budget for connectivity but requires uninterrupted business operations. As a network engineer, recommend a suitable internet connection, networking devices, and security mechanisms. Justify your recommendations by considering bandwidth limitations, reliability, cost, and future scalability

Answer:

1. Understanding the Constraints

- No wired broadband available in the rural location.

- Must support cloud applications, video conferencing, and secure HQ communication - all bandwidth- and latency-sensitive.

- Limited budget, but the business needs uninterrupted (highly available) connectivity.

- Solution must be scalable as the branch grows.

2. Recommended Internet Connectivity

- Primary link: Satellite broadband (e.g., VSAT/LEO satellite service) or a 4G/5G fixed-wireless connection, since traditional wired last-mile internet is unavailable in the area.

- Secondary/backup link: a second, independent 4G/5G LTE connection from a different provider, used for automatic failover - this directly addresses the 'uninterrupted operations' requirement without the cost of laying wired infrastructure.

- A dual-WAN router/SD-WAN-capable device can bond or fail over between the two links, giving redundancy at a fraction of the cost of a dedicated leased line.

3. Networking Devices

- Dual-WAN / SD-WAN router - manages load balancing and automatic failover between the two internet links.

- Firewall (UTM appliance) - combines routing, firewall, and VPN functions in a single cost-effective box, appropriate for a small branch budget.

- Managed switch - to segment office LAN traffic (VoIP/video conferencing on one VLAN, general data on another) for consistent call quality.

- Wireless access point - for in-office device connectivity without additional cabling cost.

4. Security Mechanisms

- Site-to-site VPN (IPsec) between the branch and headquarters to secure all inter-office traffic.

- Firewall rules restricting inbound traffic and permitting only required outbound services (cloud apps, VPN, video conferencing).

- WPA3 encryption on the wireless network and role-based access control for staff.

- Cloud application traffic secured via HTTPS/TLS, reinforced with a secure DNS/web-filtering service to block malicious sites given limited on-site IT support.

5. Justification Against the Constraints

- Bandwidth: satellite/4G-5G links provide sufficient bandwidth for video conferencing and cloud apps without requiring wired infrastructure that isn't available.

- Reliability: dual-WAN failover directly satisfies the 'uninterrupted business operations' requirement even if one link fails.

- Cost: this design avoids the high capital cost of extending wired broadband to a remote site, using commodity wireless links and a single multi-function UTM device instead of several separate appliances.

- Scalability: SD-WAN and the UTM firewall can accommodate additional links, VLANs, or bandwidth upgrades as the branch grows, without replacing the core architecture.

- A university plans to provide campus-wide wireless internet access for more than 3,000 students across classrooms, laboratories, libraries, and hostels. The network must support a large number of simultaneous users while minimizing signal interference and ensuring secure access. Due to budget constraints, only a limited number of wireless access points can be deployed. Analyze the requirements and propose an efficient wireless network design, including access point placement, frequency band selection, and authentication methods. Explain how your solution balances coverage, performance, security, and cost.

Answer:

1. Understanding the Constraints

- Must serve 3,000+ students across classrooms, labs, libraries, and hostels.

- Large number of simultaneous users -> risk of interference and congestion.

- Limited number of access points (APs) can be deployed - budget-constrained.

- Needs secure authentication for campus users.

2. Access Point Placement Strategy

- Prioritise high-density zones first: lecture halls, libraries, and hostel common areas, where the concurrent-user count per square metre is highest.

- Use ceiling-mounted, high-capacity APs (supporting Wi-Fi 6 / 802.11ax with MU-MIMO and OFDMA) rather than more numerous low-capacity APs, since Wi-Fi 6 allows more simultaneous client connections per AP - directly addressing the limited-AP-count constraint.

- Stagger AP placement in a honeycomb pattern with overlapping but non-adjacent channels to ensure continuous coverage while minimising co-channel interference.

- In hostels, place APs per floor/wing rather than per room to balance coverage against the AP budget.

3. Frequency Band Selection

- Dual-band operation (2.4 GHz and 5 GHz), with new client devices steered to 5 GHz (and 6 GHz where Wi-Fi 6E hardware is used) using band-steering.

- 5 GHz/6 GHz bands offer more non-overlapping channels than 2.4 GHz, which is critical in a high-density campus to reduce interference between adjacent APs.

- 2.4 GHz is retained only for range/legacy-device support in less dense outdoor areas.

4. Authentication Methods

- WPA2-Enterprise / WPA3-Enterprise with 802.1X authentication against the university's RADIUS server, tied to student/staff credentials - avoids a single shared password that is difficult to secure at this scale.

- eduroam-style federated authentication can be layered on top for visiting researchers/students, if applicable.

- A separate, rate-limited guest SSID with captive portal authentication for visitors, isolated on its own VLAN from the academic network.

5. Justification: Balancing Coverage, Performance, Security, and Cost

- Coverage: prioritised, high-density AP placement in classrooms/library/hostels ensures the largest population of concurrent users is served first within a limited AP budget.

- Performance: Wi-Fi 6 APs with MU-MIMO/OFDMA and 5 GHz-first band steering let each AP serve substantially more simultaneous users than legacy hardware, reducing the number of APs required to meet demand.

- Security: 802.1X/RADIUS-based enterprise authentication ties network access to individual identities, satisfying the 'secure access' requirement without added per-user hardware cost.

- Cost: investing in fewer, higher-capacity APs plus a shared RADIUS/authentication backend is more cost-effective at this scale than deploying many low-capacity APs, directly respecting the stated AP-count constraint.

- A financial institution is upgrading its internal computer network to improve cybersecurity while allowing employees to access sensitive customer information and online banking services. The organization must comply with strict security policies without significantly affecting network performance or increasing operational costs. Design a secure network architecture by selecting appropriate network segmentation techniques, firewall deployment, authentication methods, and intrusion detection mechanisms. Justify your design by considering the constraints of security, performance, compliance, and budget.

Answer:

1. Understanding the Constraints

- Must strengthen cybersecurity for sensitive customer data and online banking.

- Must comply with strict regulatory/security policies (e.g., PCI-DSS-style requirements).

- Cannot significantly degrade network performance or raise operational cost.

2. Network Segmentation

- Segment the network into VLANs by function and sensitivity: (a) core banking/transaction servers, (b) employee workstations, (c) customer-facing online banking DMZ, (d) guest/administrative Wi-Fi.

- Place customer-facing online banking servers in a DMZ, isolated from the internal core banking VLAN by an internal firewall, so a compromise of the public-facing systems cannot directly reach sensitive back-end data.

- Segmentation is a low-cost, configuration-based control (using existing switches) rather than new hardware, which keeps costs down while meeting compliance expectations for isolating sensitive data.

3. Firewall Deployment

- Perimeter firewall (or Next-Generation Firewall/NGFW) at the internet edge, enforcing default-deny inbound rules and application-aware filtering for banking traffic.

- Internal firewall between the DMZ and the core banking VLAN to control east-west traffic and prevent lateral movement if the DMZ is breached.

- NGFW capabilities (deep packet inspection, TLS inspection) allow strict policy enforcement without adding separate dedicated appliances, controlling cost.

4. Authentication Methods

- Multi-factor authentication (MFA) for employee access to sensitive customer data and administrative systems.

- Role-based access control (RBAC) so employees only access the systems/data required for their role, supporting least-privilege compliance requirements.

- Strong customer authentication (e.g., MFA/OTP) for online banking sessions, consistent with typical financial-sector regulatory requirements.

5. Intrusion Detection / Prevention

- Network-based IDS/IPS at the perimeter and DMZ boundary to detect and block known attack signatures and anomalous traffic in real time.

- Centralised logging/SIEM correlation of firewall, IDS, and authentication logs, which supports both threat detection and the audit trail required for regulatory compliance.

6. Justification Against the Constraints

- Security: layered segmentation, firewalls, MFA, and IDS/IPS together form defence-in-depth around the most sensitive systems (core banking data).

- Performance: segmentation and NGFW policies are targeted at trust boundaries rather than inspecting all internal traffic uniformly, limiting the performance overhead; placing IDS/IPS only at key boundaries (perimeter, DMZ) avoids bottlenecking internal traffic.

- Compliance: DMZ isolation, RBAC, MFA, and centralised logging directly map to common financial-sector regulatory controls (data isolation, access control, auditability).

- Budget: the design reuses existing switching infrastructure for VLAN segmentation and consolidates firewall/IDS functions on NGFW appliances rather than requiring many single-purpose devices, keeping capital and operational costs contained.

RUBRICS FOR CALCULATION

| Criteria | Weightage (%) | Exemplary (4) | Proficient (3) | Developing (2) | Beginning (1) |
| --- | --- | --- | --- | --- | --- |
| Understanding of Problem & Constraints | 20% | Clearly identifies all constraints and fully understands the problem requirements | Identifies most constraints with minor gaps in understanding | Identifies some constraints but misses key aspects | Shows little or no understanding of constraints or problem |
| Problem Decomposition | 15% | Breaks problem into logical, well-structured sub-problems effectively | Breaks problem into sub-parts with some logical flow | Attempts decomposition but lacks clarity or structure | No clear decomposition of the problem |
| Application of Constraints in Solution | 20% | Applies all constraints correctly and consistently in solution design | Applies most constraints correctly with minor errors | Applies some constraints but with noticeable errors | Fails to apply constraints appropriately |
| Logical Reasoning & Strategy | 15% | Uses strong, clear, and efficient reasoning strategies | Uses generally sound reasoning with minor gaps | Reasoning is partially correct but inconsistent | Reasoning is unclear or incorrect |
| Solution Accuracy & Feasibility | 15% | Solution is fully correct, feasible, and optimized | Solution is mostly correct and feasible | Solution has errors or is partially feasible | Solution is incorrect or not feasible |
| Creativity & Innovation | 5% | Demonstrates highly innovative and unique approach | Shows some creativity in approach | Limited creativity shown | No creativity; relies on basic or copied ideas |
| Clarity of Presentation | 10% | Solution is clearly presented with excellent organization and explanation | Presentation is clear with minor issues | Presentation lacks clarity or organization | Poorly presented and difficult to understand |