SIMATS ENGINEERING

ASSESSMENT TOOL 2

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO4: 
Design network topologies star, mesh, bus, and hybrid using networking devices, transmission media, and cabling standards

(BL2,BL6)

Assessment Tool Weightage: 30%

Annexure A

Industry Problem Solving Task

Code of Conduct:

I Sri Sayee K(192421062) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

ksrisayee

Annexure A

Industry Problem Solving Task

- A star topology is ideal for a 20-person startup because all devices connect to a central switch, making the network easy to manage and troubleshoot. At the core sits a Layer 2/Layer 3 managed switch (such as a Cisco Catalyst or Netgear ProSafe) with at least 24 ports to accommodate all workstations, printers, and VoIP phones with room to grow. Each device connects to the switch using Cat6 UTP cables, which support Gigabit speeds up to 100 meters — more than sufficient for any typical office floor plan. The switch uplinks to a router/firewall (such as a Cisco RV series or pfSense appliance) that handles internet access, DHCP, and network address translation. A Wi-Fi access point connected to the switch extends wireless coverage for laptops and mobile devices. The key advantage of this design is fault isolation: if one workstation's cable fails, only that single node goes offline, leaving all other 19 employees unaffected. Centralised cable management also simplifies future moves, adds, and changes.

Answer: Confirmed design summary: Star topology, central 24-port managed switch (e.g. Cisco Catalyst 2960), Cat6 UTP drops to each workstation/VoIP phone (<=100m), a Wi-Fi AP for wireless coverage, and a router/firewall uplink for internet, DHCP, and NAT. This satisfies the brief because a single-point client failure (a cut cable or dead NIC) only isolates that one PC, cabling and moves/adds/changes stay centralized at the wiring closet, and 24 ports comfortably covers 20 users plus headroom for a printer and a couple of future hires -- all at low hardware cost, which matches a startup's budget constraints.

- A bank demands near-zero downtime, making a full or partial mesh topology the right choice. In a full mesh, every core network device — routers, core switches, and servers — connects directly to every other device, so no single link or node failure can bring down the network. For a bank, a partial mesh is typically deployed at the core layer (connecting all core switches and firewalls to each other) while the distribution and access layers use redundant uplinks to at least two core switches. Dual enterprise-grade routers (such as Cisco ASR or Juniper MX series) provide BGP-based redundant internet connectivity. All links between core devices use fiber optic cables — either single-mode for long distances between buildings or multi-mode OM4 within the data center — because fiber is immune to electromagnetic interference, supports very high bandwidth, and is essential for maintaining low-latency financial transactions. Redundant power supplies, UPS systems, and RAID storage arrays further complement the mesh design. The justification is straightforward: when a link fails in a mesh, traffic automatically reroutes through alternative paths in milliseconds using protocols like OSPF or BGP, satisfying regulatory requirements for uptime and data integrity.

Answer: Confirmed design summary: partial mesh at the core (all core switches/firewalls fully interconnected), redundant dual-ISP routers running BGP, and at least dual uplinks from distribution/access layers into two separate core switches. All core-to-core and inter-building links use fiber (single-mode between buildings, multi-mode OM4 within the data center) for EMI immunity and low latency. OSPF/BGP provide sub-second automatic rerouting on link failure. This design directly satisfies a bank's near-zero-downtime and regulatory-compliance requirements: there is no single point of failure at the core, failover is automatic rather than manual, and fiber plus redundant power/UPS/RAID protect both availability and data integrity end to end.

- For a small classroom lab with a tight budget, bus topology offers the lowest hardware cost because all computers share a single common communication line — the bus — eliminating the need for a central switch or hub. A single coaxial cable (RG-58, 50-ohm) or, in a modern variation, a single Cat5e/Cat6 cable daisy-chained through an inexpensive unmanaged hub runs along the length of the room. Each computer connects to the backbone using a T-connector and a BNC connector in a classic coaxial setup, with terminators (50-ohm) placed at both ends of the cable to absorb signals and prevent reflection. For a more contemporary low-cost implementation, a single inexpensive 8- or 16-port unmanaged switch replaces the terminators and uses Cat5e UTP patch cables to each workstation — achieving bus-like simplicity at minimal cost. The primary limitation is that the network is susceptible to collisions and a single cable break disrupts all connected machines, but for a supervised classroom environment with light usage (internet browsing, file sharing), this trade-off is entirely acceptable. No dedicated server is required; a teacher's PC can share resources using built-in Windows/Linux file sharing.

Answer: Confirmed design summary: bus topology (or its modern low-cost equivalent) using a single RG-58 coax backbone with BNC T-connectors and 50-ohm terminators at both ends -- or, more practically today, one inexpensive 8/16-port unmanaged switch with Cat5e patch cables to each PC. No dedicated server or managed switch is needed; the teacher's PC handles file sharing natively. This meets the brief's core constraint -- minimum cost -- while accepting the known trade-offs (collision domain shared by all nodes, one cable break can disrupt the segment), which are reasonable for a small, supervised, low-traffic classroom rather than a production network.

- A large university campus serves thousands of users across multiple buildings — lecture halls, labs, libraries, dormitories, and administration — making a single topology inadequate. A hybrid topology combines the best elements of multiple designs: a fiber-based ring or mesh at the campus backbone level for redundancy, a star topology within each building for manageability, and bus or tree structures within individual labs for cost-efficiency. At the campus core, high-speed fiber optic cables (single-mode, 10 Gbps or 40 Gbps) interconnect core routers and multilayer switches using a ring or partial mesh arrangement, ensuring that no single link failure isolates an entire building. Each building has its own distribution switch connected to the campus core via dual fiber uplinks. Within each building, a traditional star topology branches from the distribution switch to floor-level access switches, which then connect individual devices via Cat6 cables. This hierarchical three-layer model — core, distribution, and access — is the industry standard for enterprise and campus networks because adding a new building simply means installing a new distribution switch and running fiber to the core, with no disruption to existing infrastructure. Wireless access points throughout the campus connect via PoE (Power over Ethernet) to access switches, eliminating the need for separate power cabling. The hybrid design thus delivers redundancy at the backbone, simplicity at the building level, and scalability campus-wide.

Answer: Confirmed design summary: three-layer hierarchical hybrid -- a fiber ring/partial-mesh backbone (10-40 Gbps single-mode) linking core routers and multilayer switches between buildings, dual fiber uplinks from each building's distribution switch to the core, and star topology within each building cascading from distribution to floor-access switches to end devices over Cat6. PoE powers wireless APs at the access layer. This satisfies a campus's scale and reliability needs because backbone redundancy prevents any single building from being isolated by one link failure, the star wiring within buildings keeps day-to-day management and troubleshooting simple, and expansion (a new building) only requires a new distribution switch and a fiber run to the core, with zero disruption to the rest of the campus.

- Designing a network for a company with multiple departments involves creating a structured and scalable architecture using switches and routers to ensure efficient communication and security. Each department (such as HR, Finance, Sales, and IT) is connected through dedicated Layer 2 switches that form the access layer, allowing devices like computers and printers to communicate within the same department. These switches are then connected to a central Layer 3 switch or router, which acts as the core layer and enables communication between different departments through inter-VLAN routing. Virtual LANs (VLANs) are configured to logically separate each department’s network, improving security and reducing broadcast traffic. A router is also used to connect the internal network to external networks such as the internet, often with firewall functionality for security. Redundant links and backup paths can be included to ensure reliability, while proper IP addressing and subnetting are applied for efficient network management. This hierarchical design improves performance, enhances security, and allows easy expansion as the company grows.

Answer: Confirmed design summary: per-department Layer 2 access switches feeding a central Layer 3 switch/router that performs inter-VLAN routing, with one VLAN per department (HR, Finance, Sales, IT), a firewall-enabled router for internet egress, redundant core links for reliability, and a structured IP addressing/subnetting scheme (e.g. 10.0.x.0/24 per VLAN). This satisfies a multi-department company's needs because VLAN segmentation contains broadcast traffic and limits lateral access between departments (e.g. Finance traffic stays isolated from Sales), inter-VLAN routing at the core still allows controlled cross-department communication, and adding a new department later just means provisioning a new VLAN and access switch port -- no re-cabling of the whole network.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Exemplary (4) | Proficient (3) | Developing (2) | Beginning (1) |
| --- | --- | --- | --- | --- | --- |
| Problem Analysis | 15% | Clearly identifies and deeply analyzes the industry problem with all factors | Good understanding with minor gaps | Basic understanding; some key aspects missing | Poor or incorrect understanding of the problem |
| Solution Design | 20% | Innovative, practical, and industry-relevant solution | Effective solution with minor limitations | Partially suitable solution | Inappropriate or unclear solution |
| Technical Implementation | 20% | Fully functional, accurate, and well-executed implementation | Mostly functional with minor errors | Partially implemented solution | Incomplete or non-functional implementation |
| Use of Tools & Technologies | 10% | Appropriate and advanced use of relevant tools and technologies | Correct use of tools | Limited or basic use of tools | Incorrect or no use of tools |
| Problem-Solving Approach | 10% | Logical, systematic, and well-structured approach | Mostly logical approach | Somewhat disorganized approach | No clear approach |
| Innovation & Creativity | 10% | Highly creative and original solution | Some creativity | Limited originality | No creativity |
| Testing & Validation | 5% | Thorough testing with real-world scenarios | Adequate testing | Minimal testing | No testing |
| Documentation & Presentation | 5% | Clear, professional, and well-documented | Good documentation | Basic documentation | Poor or no documentation |
| Teamwork / Collaboration | 5% | Excellent coordination and contribution (if team-based) | Good collaboration | Limited participation | No collaboration or contribution |