SIMATS ENGINEERING

ASSESSMENT TOOL 2

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA0706

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30%

Annexure A

Scenario-Based

Code of Conduct:

I Sri Sayee K(192421062) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

ksrisayee

Annexure A

Scenario-Based

1. A newly established Smart Campus is deploying a secure network infrastructure for its academic and administrative departments. The network administrator has been allocated the IP address block 192.168.100.0/24. Based on the expected number of devices, the following subnet masks have already been assigned:

School of Computing → /25

School of Electronics → /26

School of Mechanical Engineering → /27

As the network engineer, design the subnet allocation by performing the following tasks:

- Determine the network address and broadcast address for each department.

- Identify the valid host IP address range for each subnet.

- Calculate the number of usable host addresses available in each subnet.

- Determine the remaining unused IP address range within the given network block.

Given

Network Address: 192.168.100.0/24

Allocated Subnets:

- School of Computing → /25

- School of Electronics → /26

- School of Mechanical Engineering → /27

Determine:

- Network Address

- Broadcast Address

- Valid Host Range

- Usable Hosts

- Remaining Unused IP Range

Method

Formula 1: Total Addresses

232-Prefix

Formula 2: Usable Hosts

232-Prefix-2

Subnet Allocation

The network 192.168.100.0/24 contains 256 IP addresses (0–255).

The assigned subnets are allocated sequentially.

A. School of Computing (/25)

Prefix

/25

Total Addresses

27=128

Usable Hosts

128-2=126

Network Address

192.168.100.0

Broadcast Address

192.168.100.127

Valid Host Range

192.168.100.1

to

192.168.100.126

B. School of Electronics (/26)

Next available block starts at 128.

Total Addresses

26=64

Usable Hosts

64-2=62

Network Address

192.168.100.128

Broadcast Address

192.168.100.191

Valid Host Range

192.168.100.129

to

192.168.100.190

C. School of Mechanical Engineering (/27)

Next available block starts at 192.

Total Addresses

25=32

Usable Hosts

32-2=30

Network Address

192.168.100.192

Broadcast Address

192.168.100.223

Valid Host Range

192.168.100.193

to

192.168.100.222

Remaining Unused Address Range

Addresses used

- /25 → 0–127

- /26 → 128–191

- /27 → 192–223

Remaining Addresses

192.168.100.224

to

192.168.100.255

These 32 addresses remain unallocated and can be reserved for future subnet expansion.

Final Answer

| Department | Subnet | Network Address | Host Range | Broadcast Address | Usable Hosts |
| --- | --- | --- | --- | --- | --- |
| School of Computing | /25 | 192.168.100.0 | 192.168.100.1 – 192.168.100.126 | 192.168.100.127 | 126 |
| School of Electronics | /26 | 192.168.100.128 | 192.168.100.129 – 192.168.100.190 | 192.168.100.191 | 62 |
| School of Mechanical Engineering | /27 | 192.168.100.192 | 192.168.100.193 – 192.168.100.222 | 192.168.100.223 | 30 |

Unused IP Address Range

192.168.100.224 – 192.168.100.255

2. A multinational software company is expanding its headquarters and has been assigned the private IP block 10.10.0.0/16. Different departments require different subnet sizes based on the number of employees. The network administrator has allocated the following subnet masks:

Software Development → /24

Quality Assurance → /25

Human Resources → /26

Executive Management → /27

As the network administrator, perform the following tasks:

- Determine the network address and broadcast address for each department.

- Identify the valid range of host IP addresses.

- Calculate the number of usable hosts in each subnet.

- Identify the unused IP address range remaining within the allocated IP block.

Given

Network Address: 10.10.0.0/16

Allocated Subnets:

- Software Development → /24

- Quality Assurance → /25

- Human Resources → /26

- Executive Management → /27

Determine:

- Network Address

- Broadcast Address

- Valid Host Range

- Usable Hosts

- Remaining Unused IP Address Range

Method

Formula 1: Total Addresses

232-Prefix

Formula 2: Usable Hosts

232-Prefix-2

The allocated subnets are assigned sequentially within the 10.10.0.0/16 network.

A. Software Development (/24)

Prefix

/24

Total Addresses

28=256

Usable Hosts

256-2=254

Network Address

10.10.0.0

Broadcast Address

10.10.0.255

Valid Host Range

10.10.0.1

to

10.10.0.254

B. Quality Assurance (/25)

Next available subnet starts at 10.10.1.0

Total Addresses

27=128

Usable Hosts

128-2=126

Network Address

10.10.1.0

Broadcast Address

10.10.1.127

Valid Host Range

10.10.1.1

to

10.10.1.126

C. Human Resources (/26)

Next available subnet starts at 10.10.1.128

Total Addresses

26=64

Usable Hosts

64-2=62

Network Address

10.10.1.128

Broadcast Address

10.10.1.191

Valid Host Range

10.10.1.129

to

10.10.1.190

D. Executive Management (/27)

Next available subnet starts at 10.10.1.192

Total Addresses

25=32

Usable Hosts

32-2=30

Network Address

10.10.1.192

Broadcast Address

10.10.1.223

Valid Host Range

10.10.1.193

to

10.10.1.222

Remaining Unused IP Address Range

Within the portion allocated sequentially:

10.10.1.224

to

10.10.255.255

These addresses remain available for future subnet allocation within the 10.10.0.0/16 network.

Final Answer

| Department | Subnet | Network Address | Valid Host Range | Broadcast Address | Usable Hosts |
| --- | --- | --- | --- | --- | --- |
| Software Development | /24 | 10.10.0.0 | 10.10.0.1 – 10.10.0.254 | 10.10.0.255 | 254 |
| Quality Assurance | /25 | 10.10.1.0 | 10.10.1.1 – 10.10.1.126 | 10.10.1.127 | 126 |
| Human Resources | /26 | 10.10.1.128 | 10.10.1.129 – 10.10.1.190 | 10.10.1.191 | 62 |
| Executive Management | /27 | 10.10.1.192 | 10.10.1.193 – 10.10.1.222 | 10.10.1.223 | 30 |

Unused IP Address Range

10.10.1.224 – 10.10.255.255

3.A smart hospital is migrating its network infrastructure from IPv4 to IPv6 to support thousands of connected medical devices and IoT sensors. The hospital has been assigned the IPv6 network:

2001:0db8:abcd:0000:0000:0000:0000:0000/64

As the network engineer responsible for the migration, perform the following:

- Convert the given IPv6 address into its compressed notation.

- Expand the compressed address back into its full hexadecimal representation.

- Identify the network prefix using the /64 prefix length.

- Calculate the total number of host addresses supported within the subnet.

- the total number of possible host addresses available within this network.

Given

IPv6 Network Address:

2001:0db8:abcd:0000:0000:0000:0000:0000/64

Tasks:

- Convert the IPv6 address into compressed notation.

- Expand the compressed address back to full notation.

- Identify the network prefix using /64.

- Calculate the total number of host addresses supported in the subnet.

Method

IPv6 addresses are 128 bits long and written as 8 hexadecimal blocks (16 bits each).

Compression rules:

- Remove leading zeros in each block.

- Replace one continuous sequence of zero blocks with ::.

With a /64 prefix, the:

- First 64 bits represent the network prefix.

- Remaining 64 bits represent the host/interface ID.

1. Compress the IPv6 Address

Original Address:

2001:0db8:abcd:0000:0000:0000:0000:0000

After removing leading zeros and replacing consecutive zero blocks:

Compressed Address = 2001:db8:abcd::

2. Expand the IPv6 Address

Expanding the compressed notation by restoring the omitted zero blocks:

Expanded Address = 2001:0db8:abcd:0000:0000:0000:0000:0000

3. Determine the Network Prefix

Given Prefix Length = /64

The first four hexadecimal blocks (64 bits) form the network portion.

Network Prefix =

2001:0db8:abcd:0000::/64

4. Calculate the Total Host Addresses

Total IPv6 bits = 128

Network bits = 64

Host bits:

128-64=64

Number of host addresses:

264
=18,446,744,073,709,551,616

Total Host Addresses = 18,446,744,073,709,551,616 (2⁶⁴)

Final Answer

| Parameter | Result |
| --- | --- |
| Compressed IPv6 Address | 2001:db8:abcd:: |
| Expanded IPv6 Address | 2001:0db8:abcd:0000:0000:0000:0000:0000 |
| Network Prefix | 2001:0db8:abcd:0000::/64 |
| Host Bits | 64 |
| Total Host Addresses | 2⁶⁴ = 18,446,744,073,709,551,616 |

4. An Internet Service Provider (ISP) connects multiple customer networks through a core router. The router maintains the following routing table entries:

192.168.1.0/24

192.168.2.0/24

192.168.3.0/24

A packet arrives at the router with the destination IP address:

192.168.2.45

As the routing engineer, determine:

- The destination subnet to which the packet belongs.

- The matching routing table entry.

- The next-hop network/interface used to forward the packet.

- The default route that should be used if the destination does not match any routing table entry.

Given

The router contains the following routing table entries:

- 192.168.1.0/24

- 192.168.2.0/24

- 192.168.3.0/24

A packet arrives with the destination IP address:

192.168.2.45

Determine:

- Destination subnet

- Matching routing table entry

- Next-hop network/interface

- Default route if no match exists.

Method

A router forwards packets using Longest Prefix Match. It compares the destination IP address with all routing table entries and selects the subnet that matches the destination network. If no entry matches, the router forwards the packet using the default route.

1. Determine the Destination Subnet

Given Destination IP:

192.168.2.45

Subnet Mask:

/24 = 255.255.255.0

The first three octets (192.168.2) represent the network portion.

Destination Subnet = 192.168.2.0/24

2. Matching Routing Table Entry

Routing Table:

- 192.168.1.0/24

- 192.168.2.0/24 ✔

- 192.168.3.0/24

The destination IP 192.168.2.45 belongs to the subnet 192.168.2.0/24.

Matching Routing Entry = 192.168.2.0/24

3. Next-Hop Network / Interface

Since the destination matches 192.168.2.0/24, the router forwards the packet through the interface (or next hop) configured for that subnet.

Next-Hop Network/Interface = Interface connected to 192.168.2.0/24

4. Default Route

If the destination IP does not match any routing table entry, the router forwards the packet through the default route.

The default route is represented as:

0.0.0.0/0

This route directs packets to the default gateway for unknown destinations.

Final Answer

| Task | Answer |
| --- | --- |
| Destination Subnet | 192.168.2.0/24 |
| Matching Routing Entry | 192.168.2.0/24 |
| Next-Hop Network/Interface | Interface connected to 192.168.2.0/24 |
| Default Route | 0.0.0.0/0 (Default Gateway) |

5. A company establishes two office departments connected through a Layer-3 router. The allocated subnet information is as follows:

Administration LAN → 192.168.10.0/26

Finance LAN → 192.168.10.64/26

The router interfaces are configured with:

Administration Gateway → 192.168.10.1

Finance Gateway → 192.168.10.65

As the network administrator, perform the following tasks:

- Determine the network address and broadcast address for each LAN.

- Identify the valid host IP address range.

- Suggest suitable IP addresses for three computers in each LAN.

- Identify the default gateway that hosts in each LAN should use for communication.

Given

Two office departments are connected through a Layer-3 router.

- Administration LAN → 192.168.10.0/26

- Finance LAN → 192.168.10.64/26

Router Interfaces:

- Administration Gateway → 192.168.10.1

- Finance Gateway → 192.168.10.65

Determine:

- Network Address and Broadcast Address

- Valid Host IP Range

- Suitable IP addresses for three computers in each LAN

- Default Gateway for each LAN

Method

A /26 subnet provides:

- Total Addresses = 232-26=64

- Usable Hosts = 64-2=62

The first address is the Network Address, the last address is the Broadcast Address, and the remaining addresses are assigned to hosts.

1. Administration LAN (192.168.10.0/26)

Network Address: 192.168.10.0

Broadcast Address: 192.168.10.63

Valid Host Range:

192.168.10.1 – 192.168.10.62

2. Finance LAN (192.168.10.64/26)

Network Address: 192.168.10.64

Broadcast Address: 192.168.10.127

Valid Host Range:

192.168.10.65 – 192.168.10.126

3. Suggested IP Addresses

Administration LAN

- Computer 1 → 192.168.10.2

- Computer 2 → 192.168.10.3

- Computer 3 → 192.168.10.4

Finance LAN

- Computer 1 → 192.168.10.66

- Computer 2 → 192.168.10.67

- Computer 3 → 192.168.10.68

4. Default Gateway

- Administration LAN: 192.168.10.1

- Finance LAN: 192.168.10.65

These gateway addresses allow hosts in each LAN to communicate with devices in other networks through the Layer-3 router.

Final Answer

| LAN | Network Address | Host Range | Broadcast Address | Default Gateway |
| --- | --- | --- | --- | --- |
| Administration | 192.168.10.0 | 192.168.10.1 – 192.168.10.62 | 192.168.10.63 | 192.168.10.1 |
| Finance | 192.168.10.64 | 192.168.10.65 – 192.168.10.126 | 192.168.10.127 | 192.168.10.65 |

Suggested IP Addresses

| Administration LAN | Finance LAN |
| --- | --- |
| 192.168.10.2 | 192.168.10.66 |
| 192.168.10.3 | 192.168.10.67 |
| 192.168.10.4 | 192.168.10.68 |

RUBRICS FOR CALCULATION

| Scenario Based Assessment |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Criteria | Weightage (%) | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| Understanding of Scenario | 20% | Demonstrates clear and complete understanding of the scenario and context | Shows good understanding with minor gaps | Partial understanding; misses key aspects | Misinterprets or fails to understand the scenario |
| Identification of Key Issues | 20% | Accurately identifies all relevant issues and factors involved | Identifies most key issues with minor omissions | Identifies limited issues; some irrelevant points | Unable to identify key issues |
| Application of Concepts | 25% | Applies appropriate concepts effectively to address the scenario | Applies concepts with minor errors | Limited or partially correct application | Unable to apply relevant concepts |
| Decision Making | 20% | Makes well-reasoned, logical decisions with strong rationale | Makes reasonable decisions with some justification | Makes weak decisions with limited justification | No clear decisions made or rationale provided |
| Clarity of Response | 15% | Response is clear, structured, and logically presented | Generally clear with minor issues | Somewhat unclear or poorly structured | Disorganized and difficult to understand |