SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE NAME:	CSA07 COURSE CODE: Computer Networks

COURSE OUTCOME COVERED

CO1: Apply the functions of OSI layer in enabling reliable data transmission across networks. (BL2, BL3)

Assessment Tool Weightage: 40%

Annexure A

SHORT ANSWER TEST

Dr.V.Parthipan

Code of Conduct:

I ____________________ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

Annexure A

Short Answer Test

1. A company needs to transfer a 150 MB design file from a client computer to a remote server over a network with a bandwidth of 50 Mbps. The Transport Layer divides the file into 1500-byte segments, while the Data Link Layer encapsulates each segment into frames by adding 40 bytes of header and trailer. Calculate the total number of segments and frames transmitted, the total transmission overhead introduced by the Data Link Layer, and estimate the total transmission time (ignoring propagation delay). Explain how the Transport and Data Link layers ensure reliable communication.

Ans)

Given

- File size = 150 MB

- Bandwidth = 50 Mbps

- Segment size = 1500 bytes

- Frame overhead = 40 bytes

Calculation

File size in bytes:

150×1024×1024=157,286,400 bytes

Number of Segments

157,286,400÷1500=104,858 segments

Each segment forms one frame.

Number of Frames = 104,858

Total Overhead

104,858×40=4,194,320 bytes≈4 MB

Transmission time:

150×850=24 sec (approx.)

Answer

- Segments = 104,858

- Frames = 104,858

- Overhead = 4,194,320 bytes (≈4 MB)

- Transmission Time ≈ 24 seconds

Explanation

The Transport Layer divides the file into segments for reliable delivery and retransmission if required. The Data Link Layer encapsulates each segment into a frame by adding headers and trailers for addressing and error detection (CRC), ensuring reliable communication.

2. A Transport Layer protocol uses a sliding window size of 6 frames. Each frame carries 1024 bytes of user data. If the sender transmits 48 frames, calculate the number of transmission windows required. If one frame in every window is lost and must be retransmitted, determine the total number of retransmissions. Explain how flow control improves communication efficiency.

Ans)

Given

- Window Size = 6

- Total Frames = 48

Calculation

Number of Windows

48÷6=8

One frame is lost in every window.

Retransmissions

8×1=8

Answer

- Windows Required = 8

- Retransmissions = 8

Explanation

The Sliding Window Protocol allows multiple frames to be sent before receiving acknowledgements, improving bandwidth utilization. Flow control prevents the sender from overwhelming the receiver, reducing congestion and improving communication efficiency.

3. A 12,000-byte IP packet is transmitted through a router where the Maximum Transmission Unit (MTU) is 1500 bytes. Assuming each fragment requires a 20-byte IP header, calculate the total number of fragments created and the total header overhead introduced. Explain how the Network Layer ensures successful delivery of fragmented packets to the destination.

Ans)

Given

- Packet Size = 12,000 bytes

- MTU = 1500 bytes

- IP Header = 20 bytes

Calculation

Maximum payload per fragment:

1500-20=1480 bytes

Fragments required:

12000÷1480≈9

Header overhead:

9×20=180 bytes

Answer

- Fragments = 9

- Header Overhead = 180 bytes

Explanation

The Network Layer fragments packets larger than the MTU and assigns fragment offsets and identification numbers. The destination reassembles all fragments correctly, ensuring reliable end-to-end communication.

4. A Transport Layer protocol uses a sliding window size of 6 frames. Each frame carries 1024 bytes of user data. If the sender transmits 48 frames, calculate the number of transmission windows required. If one frame in every window is lost and must be retransmitted, determine the total number of retransmissions. Explain how flow control improves communication efficiency.

Ans)

Given

- Window Size = 6

- Total Frames = 48

- One frame is lost in every window

Calculation

Number of Windows

48÷6=8

Retransmissions

8×1=8

Answer

- Windows Required = 8

- Retransmissions = 8

Explanation

Flow control ensures that the sender transmits data at a rate the receiver can handle. The Sliding Window Protocol allows multiple frames to be sent before receiving acknowledgements, improving bandwidth utilization and reducing unnecessary delays and congestion.

5. A multimedia application transfers a 600 MB video file. The Session Layer inserts a synchronization checkpoint after every 60 MB of transmitted data. During transmission, a failure occurs after 390 MB has been transferred. Determine the number of checkpoints created before failure and calculate the amount of data that must be retransmitted after recovery. Explain the importance of synchronization in the Session Layer.

Ans)

Given

- Video File = 600 MB

- Checkpoint every 60 MB

- Failure after 390 MB

Calculation

Checkpoints created before failure:

390÷60=6 checkpoints

Last checkpoint = 360 MB

Data to retransmit:

390-360=30 MB

Answer

- Checkpoints Created = 6

- Data Retransmitted = 30 MB

Explanation

The Session Layer creates synchronization checkpoints during communication. If a failure occurs, transmission resumes from the last checkpoint instead of restarting from the beginning. This saves time, bandwidth, and improves communication reliability.

6. Before transmission, the Presentation Layer compresses a 900 MB file by 40%. The compressed data is encrypted, adding 5% overhead to the compressed size. Calculate the compressed file size, the encrypted file size, and the total percentage reduction compared to the original file. Explain how the Presentation Layer improves transmission efficiency and security.

Ans)

Given

- Original File = 900 MB

- Compression = 40%

- Encryption Overhead = 5%

Calculation

Compressed Size

900×60%=540 MB

Encrypted Size

540+5%×540=540+27=567 MB

Percentage Reduction

900-567900×100=37%

Answer

- Compressed Size = 540 MB

- Encrypted Size = 567 MB

- Overall Reduction = 37%

Explanation

The Presentation Layer compresses data to reduce transmission time and bandwidth usage. It also encrypts data to ensure confidentiality and protect sensitive information during transmission, improving both efficiency and security.

7.An FTP application transfers 3 GB of data over a network with an effective throughput of 120 Mbps. The Application Layer divides the data into requests of 3 MB each. Calculate the total number of requests generated and estimate the total transmission time in seconds. Discuss how the Application Layer provides reliable services to users.

Ans)

Given

- Data Size = 3 GB = 3072 MB

- Throughput = 120 Mbps

- Request Size = 3 MB

Calculation

Number of Requests

3072÷3=1024

Transmission Time

3 GB=24 Gb
24,576 Mb120 Mbps=204.8 seconds

Answer

- Requests Generated = 1024

- Transmission Time ≈ 205 seconds

Explanation

The Application Layer provides services such as FTP for file transfer. It divides the data into requests, manages communication with the user, and ensures reliable file transfer through authentication, error reporting, and protocol support.

8.A packet experiences the following processing delays at different OSI layers: Application Layer = 4 ms, Transport Layer = 3 ms, Network Layer = 5 ms, Data Link Layer = 2 ms, and Physical Layer = 1 ms. The propagation delay is 18 ms, and the transmission delay is 12 ms. Calculate the total end-to-end delay experienced by the packet and explain how each layer contributes to the communication process.

Ans) Given

- Application Delay = 4 ms

- Transport Delay = 3 ms

- Network Delay = 5 ms

- Data Link Delay = 2 ms

- Physical Delay = 1 ms

- Propagation Delay = 18 ms

- Transmission Delay = 12 ms

Calculation

Total Delay

4+3+5+2+1+18+12=45 ms

Answer

- Total End-to-End Delay = 45 ms

Explanation

Each OSI layer contributes to communication by performing specific tasks such as application processing, reliable transport, routing, framing, and signal transmission. The propagation and transmission delays together determine the total time required for a packet to reach its destination

9.A network transfers 80 MB of useful data using frames of 1600 bytes, where each frame contains 80 bytes of protocol overhead added by different OSI layers. Calculate the total number of frames required, the total overhead transmitted, and the transmission efficiency as a percentage. Explain how protocol overhead affects network performance.

Ans)

Given

- Useful Data = 80 MB

- Frame Size = 1600 bytes

- Overhead per Frame = 80 bytes

Calculation

Useful Data in Bytes

80×1024×1024=83,886,080 bytes

Number of Frames

83,886,080÷1600≈52,429

Total Overhead

52,429×80=4,194,320 bytes

Transmission Efficiency

Useful data per frame

1600-80=1520 bytes
15201600×100=95%

Answer

- Frames Required = 52,429

- Total Overhead = 4,194,320 bytes

- Transmission Efficiency = 95%

Explanation

Protocol overhead carries addressing, control, and error-checking information. Although it slightly reduces efficiency, it is essential for reliable communication, routing, and error detection across the network.

10.A hospital transfers a patient's medical record of 240 MB through an IoT-enabled healthcare network. The Presentation Layer compresses the data by 30%, the Transport Layer divides the compressed data into 1400-byte segments, and the Data Link Layer adds 60 bytes of overhead to each frame. Calculate the compressed file size, the total number of segments transmitted, the total protocol overhead introduced, and the final amount of data transmitted across the Physical Layer. Explain how multiple OSI layers work together to ensure secure and reliable communication.

Ans)

Given

- Original File = 240 MB

- Compression = 30%

- Segment Size = 1400 bytes

- Frame Overhead = 60 bytes

Calculation

Compressed File Size

240×70%=168 MB

Compressed Size in Bytes

168×1024×1024=176,160,768 bytes

Number of Segments

176,160,768÷1400≈125,829

Protocol Overhead

125,829×60=7,549,740 bytes

Final Data Transmitted

176,160,768+7,549,740=183,710,508 bytes

Answer

- Compressed File Size = 168 MB

- Segments = 125,829

- Protocol Overhead = 7,549,740 bytes

- Final Data Transmitted = 183,710,508 bytes (≈175.2 MB)

Explanation

Multiple OSI layers work together during transmission. The Presentation Layer compresses data to reduce size, the Transport Layer divides it into segments for reliable delivery, the Data Link Layer adds frames and error detection, and the Physical Layer transmits the data as electrical or optical signals. This coordinated operation ensures secure, efficient, and reliable communication.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understanding of Concepts | 25% | Demonstrates a thorough, accurate understanding of concepts | Good understanding with minor gaps | Basic understanding with some misconceptions | Poor understanding; major misconceptions |
| Explanation | 25% | Detailed, well-explained answers with relevant examples | Clear explanation with adequate details | Limited explanation; lacks depth | Poor or unclear explanation |
| Application | 20% | Effectively applies concepts with strong analytical ability | Applies concepts with minor errors | Limited application with weak analysis | No application or incorrect analysis |
| Organization | 15% | Well-structured answers with logical flow and coherence | Organized with minor issues in flow | Basic structure, lacks clarity | Poor organization, incoherent answers |
| Accuracy | 10% | Completely accurate and covers all required points | Mostly accurate with minor omissions | Partially correct with missing points | Incorrect answers with major gaps |
| Clarity | 5% | Clear, neat, professional presentation | Understandable with minor issues | Limited clarity, readability issues | Poorly written, difficult to understand |