SIMATS ENGINEERING

ASSESSMENT TOOL 3

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO4: 
Design network topologies star, mesh, bus, and hybrid using networking devices, transmission media, and cabling standards

(BL2,BL6)

Assessment Tool Weightage: 30%

QUESTIONS: 5								Total Marks:25

Annexure A

Comparative Analysis

Code of Conduct:

I Sri Sayee K (192421062) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

ksrisayee

Annexure A

Comparative Analysis

- Star topology is generally more expensive than bus topology because it requires a central device such as a switch or hub and more cabling to connect each node individually. In contrast, bus topology uses a single backbone cable, making it low-cost and easier to install. However, in terms of reliability, star topology is superior because failure of one node does not affect others, whereas a failure in the main bus cable can bring down the entire network. Performance is also better in star topology since data transmission is managed through a central device, reducing collisions, while bus topology suffers from performance degradation as more devices are added due to increased data traffic on the shared medium.

Answer: Summary comparison -- Star vs. Bus:
Cost: Star > Bus (extra switch/hub + per-node cabling).
Installation: Bus is simpler (single backbone); Star needs more cable runs.
Reliability: Star wins -- one node/cable failing doesn't affect others; a bus backbone break takes down the whole segment.
Performance: Star wins -- the switch manages traffic and limits collisions; bus degrades as more devices share the same medium.
Verdict: Bus is cheaper and simpler to install; Star is more reliable and performs better as the network grows -- the right choice depends on whether budget or uptime/performance is the priority.

- Mesh topology offers very high fault tolerance because every node is connected to multiple other nodes, providing multiple paths for data transmission. If one link fails, data can still be routed through alternate paths without disrupting communication. In contrast, star topology has moderate fault tolerance; while failure of individual nodes does not affect the network, failure of the central hub or switch can cause the entire network to stop functioning. Therefore, mesh topology is more robust and reliable in critical environments compared to star topology.

Answer: Summary comparison -- Mesh vs. Star:
Fault tolerance: Mesh > Star -- multiple redundant paths mean a single link failure doesn't disrupt communication.
Single point of failure: Star has one (the central switch/hub); full mesh effectively has none.
Cost/complexity: Star is simpler and cheaper since Mesh requires far more links (n(n-1)/2 for full mesh).
Verdict: Mesh is the more robust choice for critical, uptime-sensitive environments (e.g. banking, data centers); Star remains preferable where cost and simplicity matter more than absolute redundancy.

- Bus topology is simple and easy to install because it uses a single backbone cable with minimal connections, making it suitable for small networks. On the other hand, mesh topology is highly complex to install as each node must be connected to every other node, requiring a large number of cables and ports. This complexity increases significantly as the number of devices grows, making mesh topology time-consuming and costly to implement compared to the straightforward setup of bus topology.

Answer: Summary comparison -- Bus vs. Mesh:
Installation complexity: Bus is simple (one backbone, minimal connections); Mesh is complex (every node wired to every other node).
Scaling cost: Bus scales cheaply; Mesh's cabling/port requirements grow quadratically with node count, making it costly and time-consuming at scale.
Best fit: Bus suits small, low-budget networks; Mesh suits small numbers of critical nodes (e.g. core routers) where full redundancy justifies the wiring overhead.
Verdict: Bus wins on installation simplicity and cost; Mesh wins on redundancy -- Mesh is impractical for large numbers of end devices for this reason.

- Hybrid topology is highly scalable because it combines two or more different topologies (such as star, mesh, or bus), allowing the network to expand flexibly according to organizational needs. New segments can be added without affecting the existing network structure. In comparison, star topology is also scalable but to a limited extent, as it depends on the capacity of the central device (switch or hub). Once the central device reaches its port limit, expansion requires additional hardware. Thus, hybrid topology provides greater scalability than star topology.

Answer: Summary comparison -- Hybrid vs. Star:
Scalability: Hybrid > Star -- new segments/topologies can be added without disrupting the existing structure, whereas Star is capped by the central switch's port count.
Flexibility: Hybrid can mix Star, Mesh, and Bus per segment to match each area's needs (e.g. Star per building, Mesh at the backbone); Star is a single fixed structure.
Complexity/cost: Star is simpler and cheaper to design and manage; Hybrid requires more planning and skilled administration.
Verdict: Hybrid is the better choice for large, growing organizations (e.g. a campus); Star remains ideal for a single office/building of fixed, moderate size.

- In terms of maintenance, star topology is relatively easy to manage because issues can be quickly identified and isolated to a specific node or cable. Mesh topology, while highly reliable, is difficult to maintain due to its complex structure and large number of connections, making troubleshooting time-consuming. Bus topology is harder to maintain than star because faults in the backbone cable can disrupt the entire network and are difficult to locate. Hybrid topology has moderate maintenance complexity; while it benefits from flexibility, managing multiple combined topologies requires careful monitoring and skilled administration.

Answer: Summary comparison -- Maintenance across all four:
Easiest: Star -- faults isolate to a single node/cable, quick to identify.
Harder: Bus -- a backbone fault is difficult to locate and disrupts the whole segment.
Hardest: Mesh -- large number of interconnections makes troubleshooting complex and time-consuming.
Moderate: Hybrid -- flexible, but combining topologies means administrators must monitor multiple structures at once, requiring more skill than Star alone.
Verdict: ranked easiest to hardest to maintain -- Star < Hybrid < Bus < Mesh.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Exemplary (4) | Proficient (3) | Developing (2) | Beginning (1) |
| --- | --- | --- | --- | --- | --- |
| Understanding of Concepts | 15% | Demonstrates complete and in-depth understanding of all concepts being compared | Good understanding with minor gaps | Basic understanding; some concepts unclear | Poor understanding of concepts |
| Comparison Depth | 20% | Provides detailed, insightful, and meaningful comparisons across multiple aspects | Adequate comparison with relevant points | Limited comparison; lacks depth | Minimal or incorrect comparison |
| Criteria Selection | 10% | Selects highly relevant and appropriate comparison parameters | Mostly relevant criteria | Some irrelevant or missing criteria | Poor or no criteria selection |
| Analytical Skills | 15% | Strong analysis with logical reasoning and clear justification | Good analysis with some justification | Basic analysis with weak reasoning | No clear analysis |
| Organization & Structure | 10% | Well-organized, clear, and logically structured | Generally organized | Somewhat disorganized | Poorly structured |
| Use of Examples / Evidence | 10% | Uses strong, relevant examples or data to support comparison | Uses some examples | Limited examples | No supporting examples |
| Clarity of Presentation | 10% | Very clear, concise, and easy to understand | Mostly clear | Somewhat unclear | Difficult to understand |
| Conclusion & Insights | 10% | Insightful conclusion with strong justification | Clear conclusion | Basic conclusion | No or weak conclusion |